ESP32 Maze-Solving Robot

Left-Wall Following with PID Centre-Correction Navigation

Firmware Version 8.1 ·  Project Report

| Field | Detail |
| --- | --- |
| Platform | ESP32 Devkit V1 (WROOM, no PSRAM), Arduino Core 3.x |
| Motor Driver | TB6612FNG dual H-bridge |
| Actuators | 2 × N20 12 V, 300 RPM gear motors |
| Sensors | 3 × HC-SR04 ultrasonic (Front, Left, Right) |
| Display | 0.96" SSD1306 OLED, I2C |
| Feedback | Active buzzer (non-blocking sequence player) |
| Power | 2S Li-ion pack |
| Navigation mode | PID centre-following (compile-time selectable; PD wall-follow fallback) |

# 1. Project Overview

This project implements an autonomous maze-solving robot built on an ESP32 microcontroller. The robot navigates unknown maze corridors using three ultrasonic distance sensors (front, left, right), a dual-motor differential drive, and a finite state machine (FSM) that governs forward travel, turning, reversing, and stuck-recovery behaviour. An OLED display renders an animated robot face that reflects the current navigation state, and a buzzer provides audible feedback for turns and error conditions.

The current release (v8.1) introduces a PID centre-following navigation mode that keeps the robot equidistant from both walls in a corridor, replacing the fixed-offset left-wall-following approach used in the prior version. Left-wall PD following is retained as an automatic fallback for one-sided corridors, and a purely cosmetic mouth-animation update (v8.1) was layered on top without touching navigation, sensing, or motor logic.

# 2. Hardware Architecture

## 2.1 Pin Assignments

| Subsystem | Signal | GPIO |
| --- | --- | --- |
| Motor Driver (TB6612FNG) | PWMA / AIN1 / AIN2 | 18 / 25 / 26 |
| Motor Driver (TB6612FNG) | PWMB / BIN1 / BIN2 | 19 / 27 / 14 |
| Motor Driver (TB6612FNG) | STBY | 13 |
| Front Ultrasonic | TRIG / ECHO | 21 / 23 |
| Left Ultrasonic | TRIG / ECHO | 4 / 22 |
| Right Ultrasonic | TRIG / ECHO | 15 / 5 |
| OLED (I2C) | SDA / SCL | 32 / 33 |
| Buzzer | Signal | 2 |

Design constraint: all ECHO pins are compile-time asserted to be below GPIO 32, since the interrupt handlers read pin state directly from the GPIO_IN_REG register for speed rather than calling digitalRead().

## 2.2 Board Compatibility Warning

The firmware is written for an ESP32-WROOM module without PSRAM. The right ultrasonic sensor uses GPIO 16 and 17 as ordinary digital I/O — on WROVER (PSRAM-equipped) boards these two pins are wired internally to the PSRAM chip-select and clock lines and cannot be reused as general-purpose GPIO. Running this firmware unmodified on a WROVER board would cause the right sensor to fail silently. Porting to a PSRAM board requires relocating TRIG_RIGHT/ECHO_RIGHT to unused pins (e.g. 5 and 15) and updating the corresponding #define values.

# 3. Bill of Materials

## 3.1 Controller

| Item | Qty |
| --- | --- |
| ESP32 DevKit V1 | 1 |

The ESP32 DevKit V1 is the project's main processor: a dual-core Xtensa LX6 microcontroller with built-in Wi-Fi/Bluetooth (unused here), hardware PWM (LEDC), and enough GPIO to drive the motor controller, three ultrasonic sensors, an I2C display, and a buzzer simultaneously. This project specifically requires the no-PSRAM WROOM variant of the DevKit V1 (see §2.2) because the right-side ultrasonic sensor uses GPIO 16/17, which are reserved for PSRAM on WROVER modules. The ESP32 runs the entire control loop — sensor scheduling, PID/PD navigation math, the FSM, OLED rendering, and buzzer sequencing — as a single non-blocking Arduino sketch.

## 3.2 Motors and Movement

| Item | Qty |
| --- | --- |
| N20 12V 600RPM Micro Metal Gear Motor | 2 |
| Wheels (compatible with N20) | 2 |
| Caster wheel | 1 |

The two N20 micro metal gear motors are the robot's drive actuators, one per side, forming a differential (skid-steer) drivetrain: the firmware drives them independently through setMotorA()/setMotorB(), so turning is achieved by spinning the two wheels at different speeds or in opposite directions rather than through a steering mechanism. N20 motors are a common small-robot choice because their integrated metal gearbox delivers usable torque at low voltage in a very compact housing. The matching wheels convert that rotation into forward motion, while the caster wheel is a free-rolling, non-driven support wheel (typically at the rear) that keeps the chassis level and balanced without adding a third drive motor or steering linkage.

## 3.3 Motor Driver

| Item | Qty |
| --- | --- |
| TB6612FNG Dual Motor Driver | 1 |

The TB6612FNG is a dual H-bridge motor driver IC that sits between the low-power ESP32 GPIO/PWM signals and the higher-current N20 motors, which the microcontroller cannot drive directly. Each motor channel (A and B) is controlled by two direction pins (AIN1/AIN2, BIN1/BIN2) plus a PWM speed input (PWMA/PWMB), letting the firmware set both direction and speed independently per wheel — this is what setMotorA()/setMotorB() and driveForward()/driveLeft()/driveRight() translate into. The STBY pin enables/disables the whole driver; the firmware holds it high at startup and uses motorStop() (zero PWM, direction pins low) rather than STBY toggling for routine stops.

## 3.4 Sensors

| Item | Qty |
| --- | --- |
| Distance Sensor (HC-SR04) | 3 |

Three HC-SR04 ultrasonic rangefinders — mounted facing front, left, and right — are the robot's only means of perceiving the maze. Each sensor measures distance by timing the echo of a 40 kHz ultrasonic pulse: the firmware issues a short trigger pulse and measures how long the echo line stays high, converting that time-of-flight into a distance in centimetres (pulseWidthToCm()). The front sensor detects an oncoming wall (deciding when to turn or reverse); the left and right sensors detect corridor width and feed the wall-following/centre-following navigation controller (§4.3). All three are read by a single round-robin scheduler (§4.1) rather than simultaneously, which avoids acoustic cross-talk between sensors.

## 3.5 Power System

| Item | Qty |
| --- | --- |
| 7.4V 2600mAh Li-ion Battery Pack (2S) | 1 |
| 2S 5A–10A Li-ion BMS Protection Board | 1 |
| LM2596 Buck Converter Module | 1 |

The 7.4V 2600mAh Li-ion pack (2 cells in series, “2S”) is the robot's sole energy source, sized to run both the N20 motors and the ESP32/sensor electronics for an extended session between charges. The 2S BMS (Battery Management System) protection board sits between the cells and the rest of the circuit, protecting the pack from over-charge, over-discharge, and over-current conditions that raw Li-ion cells cannot safely handle on their own. Because the ESP32 and its 3.3 V peripherals cannot tolerate the pack's nominal 7.4 V directly, the LM2596 buck converter steps that voltage down to a safe, regulated level for the logic side of the circuit, while the motors are typically fed from the higher unregulated pack voltage through the TB6612FNG.

## 3.6 Supporting Components

| Item | Qty |
| --- | --- |
| Power switch | 1 |
| Jumper wires | As needed |
| Connectors | As needed |

These are the small parts that make the rest of the BOM into a working circuit rather than a pile of modules: a power switch to cut battery current without unplugging anything, jumper wires for point-to-point signal and power connections between the ESP32, driver, sensors, and display, and connectors (headers, terminal blocks, JST-style battery connectors, etc.) for detachable, serviceable wiring at the battery, motors, and sensor leads.

## 3.7 Display

| Item | Qty |
| --- | --- |
| 0.96" I2C OLED Display Module | 1 |

The 0.96" SSD1306 OLED (128×64, monochrome, I2C interface) is the robot's status readout, driven over just two signal wires (SDA/SCL) plus power. Rather than plain text, the firmware uses it to render an animated robot face — eyes, eyebrows, and a mouth — whose expression changes with the current FSM state (§4.5), plus a text label naming the state and an optional live sensor-distance readout (DEBUG_OLED). Its main practical value is at the bench: it lets a developer see what the robot “thinks” is happening without a laptop or Serial monitor attached.

## 3.8 Mechanical Parts

| Item | Qty |
| --- | --- |
| Chassis | 1 |
| Motor mounts (N20 compatible) | 2 |
| Sensor mounts | 3 |
| Screws, nuts, spacers | As needed |

The chassis is the structural base plate that carries every other component; the motor mounts fix the two N20 gear motors to it at a consistent height and alignment so both wheels contact the ground evenly (critical for the differential drive to track straight), and the sensor mounts hold the three HC-SR04 units at fixed, known angles on the front/left/right faces of the robot, since the navigation math assumes each sensor is aimed perpendicular to its side of the chassis. Screws, nuts, and spacers are the general fastening hardware used to assemble the chassis, mounts, and driver/controller boards into a rigid unit.

Note: the firmware's header comment describes the drive motors as “300 RPM”, while the BOM specifies 600 RPM N20 motors. This is worth confirming against the actual motors used, since gear-motor RPM affects the turn-timing constants (TURN_MIN_MS/TURN_MAX_MS) and the base drive speed tuning.

# 4. Software Architecture

The firmware is a single non-blocking Arduino sketch (~1,560 lines) structured around four cooperating subsystems, all serviced from one loop() call per iteration:

- Sensor scheduler — round-robin, interrupt-driven ultrasonic ranging with median filtering

- Navigation controller — PID centre-follow or PD wall-follow motor correction

- Finite state machine (FSM) — governs which navigation/turn/recovery behaviour is active

- OLED face renderer + buzzer sequencer — status feedback, fully decoupled from navigation logic

No part of the main loop uses delay(); all timing is handled with millis()/micros() comparisons and phase state machines, keeping the sensor scheduler, FSM and display responsive on every iteration.

## 4.1 Ultrasonic Sensor Subsystem

Each HC-SR04 is driven by a shared round-robin scheduler (Front → Left → Right → Front) implemented as a 4-phase state machine: SPHASE_IDLE → SPHASE_TRIG_ACTIVE → SPHASE_WAITING_ECHO → SPHASE_SETTLING. A 12 µs trigger pulse is issued per sensor, and the echo's rising/falling edges are timestamped inside a CHANGE interrupt (IRAM_ATTR) using esp_timer_get_time() for microsecond resolution.

- Edge timestamps are read directly from GPIO_IN_REG inside the ISR for minimal latency.

- Echo data is exchanged between ISR and main loop through a shared struct guarded by a portMUX critical section, safe under the single-core Arduino default; a note in the source flags that porting to true dual-core FreeRTOS execution would require the ISRs to also take the mux.

- A 25 ms echo timeout prevents the scheduler from stalling if a sensor fails to respond.

- Each sensor's raw readings pass through a 5-sample rolling median filter (insertion sort) before being published as distFront / distLeft / distRight, rejecting single-sample ultrasonic noise and out-of-range readings (valid window: 2–400 cm).

## 4.2 Motor Control

Each DC motor is driven independently through the TB6612FNG driver using ESP32 LEDC PWM channels (22 kHz, 8-bit resolution — chosen above the audible range to avoid motor whine). setMotorA()/setMotorB() accept a signed speed (-255..255) and set the corresponding direction pins plus PWM duty cycle; driveForward(), driveRight(), and driveLeft() compose these into the robot's basic maneuvers, with in-place rotation achieved by driving the two wheels in opposite directions.

## 4.3 Navigation Control

Navigation logic is selected at compile time via the CENTER_MODE flag.

PID Centre-Follow (CENTER_MODE = 1, current default)

The robot steers to stay equidistant from both walls rather than hugging one wall at a fixed offset. The controller error term is centerError = distLeft − distRight: positive error (closer to the right wall) steers left, negative steers right.

correction = KP_CENTER·error + KI_CENTER·pidIntegral + KD_CENTER·(error − lastError) leftMotor  = BASE_SPEED + correction rightMotor = BASE_SPEED − correction

| Constant | Value | Role |
| --- | --- | --- |
| KP_CENTER | 3.0 | Proportional gain — primary steering authority |
| KI_CENTER | 0.05 | Integral gain — removes steady-state drift |
| KD_CENTER | 1.2 | Derivative gain — damps side-to-side oscillation |
| PID_INTEGRAL_LIMIT | 80.0 | Anti-windup clamp on the accumulated integral term |
| PID_CORRECTION_MAX | 70.0 PWM | Caps total correction so no wheel fully saturates on a sharp corridor |
| CORRIDOR_VALID_CM | 60.0 cm | Max range for a wall to be considered “visible” to the PID |

Safeguards:

- Corridor validity guard — full bilateral PID correction only runs when both walls read below CORRIDOR_VALID_CM. In a one-sided corridor the controller falls back to left-wall PD tracking; in open space (or with only the right wall visible) it drives straight, with an emergency steer if the left wall suddenly reads dangerously close.

- Integral freeze on fallback — the accumulator is not reset when the robot exits bilateral tracking, so context is preserved for when both walls reappear.

- Integral reset on state transition — every FSM state change (enterState()) calls resetPID(), preventing wind-up from one corridor from biasing the first correction in the next.

Left-Wall PD Fallback / Legacy Mode (CENTER_MODE = 0)

When centre PID is disabled, or as the automatic fallback described above, the robot tracks a fixed target distance from the left wall (LEFT_WALL_TARGET = 10 cm) using proportional-derivative control (KP_WALL = 4.0, KD_WALL = 1.5), with a hard emergency steer if the left wall closes to under LEFT_WALL_NEAR (7 cm).

## 4.4 Finite State Machine

The FSM defines seven states: FORWARD, TURN_RIGHT, TURN_LEFT, SEARCHING, STUCK, REVERSING, and TRANSITION. State transitions are driven by four threshold predicates evaluated against the filtered sensor readings, each using separated entry/exit thresholds (hysteresis) to prevent chattering at a boundary distance.

| Predicate | Condition | Purpose |
| --- | --- | --- |
| wallAhead() | distFront < 14.0 cm | Detects a wall closing off the front (turn-entry threshold) |
| frontClearExit() | distFront > 18.0 cm | Confirms front is clear before exiting a turn (turn-exit threshold) |
| rightOpen() | distRight > 18.0 cm | Right corridor considered open for a turn |
| leftOpen() | distLeft > 20.0 cm | Left wall lost / open corridor |

State summary:

- FORWARD — drives using the navigation controller (§4.3); on hitting a front wall, prefers turning right, then left, then reversing.

- TURN_RIGHT / TURN_LEFT — in-place rotation for a minimum of 300 ms (avoids over-eager exit) and a maximum of 2,500 ms, after which the robot is declared STUCK; exits once the front is clear and the minimum time has elapsed.

- SEARCHING — the initial/recovery state; drives forward at reduced speed until a wall or turn opportunity is detected.

- STUCK — entered when the front distance stays within 10 cm and unchanged (±2 cm) for 3 seconds while moving; immediately re-dispatches into REVERSING with an audible 3-beep warning.

- REVERSING — backs up for 350 ms, then re-evaluates for an open turn or continues reversing if still boxed in.

- TRANSITION — a short (80 ms) non-blocking pause with motors stopped, inserted between states so the drivetrain doesn't snap directly from one manoeuvre into another.

Every state transition passes through enterState(), which resets the PID/PD integrators and triggers any entry action (starting a turn timer, issuing a directional beep, etc.), keeping transition side effects in one place rather than scattered through the update logic.

## 4.5 OLED Status Display

The SSD1306 renders an animated robot face — eyes with idle glancing and periodic randomised blinking, eyebrows, and (as of v8.1) a mouth whose shape reflects the current FSM state: a closed smile while driving forward, a smirking tilt while turning, a flat parted “hmm” while searching, and a wide-open frown in STUCK/REVERSING. A divider line and text label beneath the face show the current state name, with an optional debug line (DEBUG_OLED flag) printing live front/left/right distances.

## 4.6 Buzzer Feedback

A non-blocking sequence player drives the buzzer through short on/off note patterns rather than blocking delay() calls: a rising two-note pattern for right turns, a falling two-note pattern for left turns, and a triple-beep pattern when the robot declares itself stuck.

# 5. Build-Time Configuration

| Flag | Default | Effect |
| --- | --- | --- |
| CENTER_MODE | 1 | 1 = PID centre-follow navigation; 0 = legacy left-wall PD follow (v7 behaviour) |
| DEBUG_OLED | 0 | 1 = show live F:/L:/R: sensor distances and state on the OLED |
| DEBUG_SERIAL | 0 | 1 = print sensor readings, state, and PID terms to Serial at ~10 Hz |

# 6. Known Limitations & Design Notes

- WROOM-only hardware target — GPIO 16/17 usage on the right sensor makes the sketch incompatible with WROVER/PSRAM boards without pin reassignment (§2.2).

- Single-core interrupt safety assumption — the ISR/critical-section pairing is documented as safe only under the default single-core Arduino execution model; a dual-core FreeRTOS port would need ISR-side mux locking.

- Tuning is bench-oriented — PID gains (KP_CENTER=3.0, KI_CENTER=0.05, KD_CENTER=1.2) are shipped as a conservative starting point for a ~10 cm corridor width at BASE_SPEED=180, intended to be tuned further on the physical maze.

- Turn timing is time-boxed, not encoder-based — TURN_MIN_MS/TURN_MAX_MS bound rotation duration rather than measuring actual heading change, so turn accuracy depends on consistent motor speed and battery voltage.

# 7. Revision History

## v7 → v8: PID Centre-Follow Navigation

- Added compile-time-selectable PID centre-following mode alongside the existing left-wall PD follower.

- Added corridor-validity gating, integral anti-windup, and freeze-on-fallback/reset-on-transition integral policy.

## v8 → v8.1: Mouth Animation

- Added a state-linked mouth animation to the OLED face (closed smile, turn smirk, searching “hmm”, stuck/reversing frown).

- Cosmetic only — no changes to navigation, sensing, FSM transitions, or motor control.